RADWARE  |  PROFESSIONAL SERVICES

# F5-to-Alteon Migration Tool

User Guide

Step-by-step instructions for converting an F5 BIG-IP (tmsh) configuration into a ready-to-apply Radware Alteon CLI configuration - on Windows, macOS, or Linux, from any form of F5 export.

| Product | @radware/f5-to-alteon (npm) - v0.5.6 |
| --- | --- |
| Repository | github.com/Radware/F5-to-Alteon-Migrator |
| Document date | 22 July 2026 |
| Audience | Network / ADC engineers performing F5-to-Alteon migrations |

Contents

1. Introduction	2

2. Requirements	2

3. Installation options	2

3.1 Zero-install with npx (recommended)	2

3.2 Global installation	2

3.3 From the GitHub repository	2

4. Supported inputs	3

5. Step-by-step migration	3

Step 1 - Convert	3

Step 2 - Review the logs	3

Step 3 - Apply on the Alteon	4

Step 4 - Verify traffic	4

6. Command reference	4

7. Route domains	4

8. Conversion coverage	4

9. How the tool was validated	5

10. Troubleshooting	5

11. Resources	6

# 1. Introduction

The F5-to-Alteon migration tool converts an F5 BIG-IP LTM configuration into Radware Alteon CLI commands. It accepts whatever export you have - a qkview or UCS support archive, an extracted archive folder, or bare configuration files - and produces an Alteon configuration file plus two review logs that account for every object it could not fully convert. Nothing is silently dropped.

Every conversion path has been validated against live Alteon appliances (34.0.x and 34.5.x), including behavioral round-trip testing against a live BIG-IP 17.5 (identical load-balancing distribution, persistence, and health behavior) and packet-level traffic tests of generated configurations. The tool converts everything Radware's legacy F5-Migration tool converted, and adds route domains (Alteon Network Segmentation or per-route-domain splitting) and wildcard/forwarding virtuals (Alteon filters).

# 2. Requirements

The only requirement is Node.js 16 or newer. There are no other dependencies - no Python, no build tools. Verify with:

node --version

If Node.js is not installed:

| Platform | Install command |
| --- | --- |
| Windows | winget install OpenJS.NodeJS.LTS (or download from nodejs.org) |
| macOS | brew install node (or download from nodejs.org) |
| Linux (Debian/Ubuntu) | sudo apt install nodejs npm |
| Linux (RHEL/CentOS) | sudo dnf install nodejs |

# 3. Installation options

All options behave identically on Windows (PowerShell or CMD), macOS Terminal, and Linux shells. Pick whichever fits your environment.

## 3.1 Zero-install with npx (recommended)

Runs the latest published version directly - nothing to install or update:

npx @radware/f5-to-alteon device.qkview -o out/

## 3.2 Global installation

Installs the f5-to-alteon command permanently:

npm install -g @radware/f5-to-alteon f5-to-alteon device.qkview -o out/

## 3.3 From the GitHub repository

For air-gapped environments or to run from source:

git clone https://github.com/Radware/F5-to-Alteon-Migrator.git cd F5-to-Alteon-Migrator/node node bin/f5-to-alteon.js device.qkview -o out/

# 4. Supported inputs

The input type is auto-detected by content, not by file extension. Any ONE of the following works:

| You have | Command example |
| --- | --- |
| A qkview support archive (multi-GB is fine; streamed) | npx @radware/f5-to-alteon device.qkview -o out/ |
| A UCS backup archive | npx @radware/f5-to-alteon backup.ucs -o out/ |
| An extracted qkview/UCS folder | npx @radware/f5-to-alteon ./extracted-dir/ -o out/ |
| Bare config files (base + main) | npx @radware/f5-to-alteon bigip_base.conf bigip.conf -o out/ |
| A single bigip.conf | npx @radware/f5-to-alteon bigip.conf -o out/ |
| A folder of configs from several devices | npx @radware/f5-to-alteon ./dump-folder/ -o out/ (lists the devices found and asks which one to convert) |

To export from the F5 side: tmsh save sys config then take /config/bigip.conf and /config/bigip_base.conf - or simply generate a qkview and feed it whole.

# 5. Step-by-step migration

## Step 1 - Convert

npx @radware/f5-to-alteon device.qkview -o out/ --name mydevice

The tool prints a one-line summary (objects converted, diagnostics count) and writes three files:

| File | Contents |
| --- | --- |
| mydevice_output.txt | The Alteon CLI configuration - this is what you apply on the Alteon. |
| mydevice_log1.txt | Manual-completion items: converted with a caveat to review (renamed IDs, port mappings, gateway targets, and similar). |
| mydevice_log2.txt | Possibly-unsupported items: F5 features with no automatic Alteon equivalent (iRules, GTM, and similar), each with guidance. |

## Step 2 - Review the logs

Open the two log files. Every entry names the F5 object, describes the issue, and suggests the course of action. Typical items: object names longer than Alteon's 32-character limit were renamed (references updated automatically), physical F5 ports need re-mapping to the target chassis, an iRule needs a manual equivalent.

## Step 3 - Apply on the Alteon

Paste the output file into an Alteon CLI session (or stream it over SSH), then:

apply save (only after verifying)

The output is ordered so that objects are always defined before they are referenced (segments before virtual servers, persistence before proxy-IP blocks). Review the result with the relevant /info and /cfg menus before saving to FLASH.

## Step 4 - Verify traffic

Confirm virtual servers serve traffic, health checks report real-server state, and persistence behaves as on the source. If the source F5 used SNAT automap, assign a dedicated proxy IP (PIP) on the relevant virtual services - the conversion log flags exactly where this is needed.

# 6. Command reference

| Option | Meaning |
| --- | --- |
| -o, --out <dir> | Output directory (default: alongside the input). |
| --name <prefix> | Output filename prefix (default: derived from the input name). |
| --rd-mode auto¦segment¦split | Route-domain handling; see Section 7. Default: auto. |

# 7. Route domains

F5 route domains (addresses such as 10.1.2.3%4) are converted, not ignored. Three modes:

- auto (default): if the route domains have non-overlapping address space, everything lands on ONE Alteon using Network Segmentation - each route domain becomes an Alteon segment with its VLANs, virtual servers get segment classifiers with Return-to-Last-Hop enabled, and a per-segment redirect filter forwards cross-segment traffic to that segment's gateway (taken from the route domain's default route). If address space overlaps, the tool automatically falls back to per-route-domain splitting.

- split: one self-contained output file per route domain, each intended for its OWN Alteon instance (vADC or VA).

- segment: force segmentation; falls back to split with a warning if route domains overlap.

Both strategies were validated on live Alteon appliances, including a packet-level cross-segment traffic test: segment-classified VIPs serve routed clients, cross-segment traffic is redirected to the segment gateway, and segment isolation matches F5 route-domain behavior.

# 8. Conversion coverage

| Area | Converted |
| --- | --- |
| Server load balancing | Nodes/reals (state, ratio to weight, connection limits), monitors (built-in mappings and custom; multi-monitor AND/OR to LOGEXP), pools/groups (LB metric, priority-group to backup, member-level overrides), virtual servers (VIP/service/port, reserved-port application coercion), persistence (client IP, cookie insert), profiles (HTTP/XFF, SSL to SSL policy, OneConnect), SNAT pools to proxy-IP network classes. |
| Wildcard / forwarding virtuals | Converted to Alteon filters - allow (IP forwarding) or redirect-to-group (pool-backed) - with source/destination/protocol matching and automatic port bindings from the virtual's VLAN restriction. |
| Route domains | Alteon Network Segmentation on a single device, or per-route-domain output splitting (Section 7). |
| Networking | VLANs (enabled, with port membership), self-IPs to interfaces, floating IPs, static routes and default gateways, trunks/LACP. |
| System | Management IP and gateway, NTP, syslog, SNMP. |
| Not auto-converted | iRules, GTM/GSLB, administrative partitions - each produces a named diagnostic with guidance instead of a silent drop. |

# 9. How the tool was validated

- 39 real production device configurations converted and staged on live Alteon 34.0.12.0 and 34.5.7.0 - zero remaining converter defects; every residual reject is environmental (platform capacity, physical ports, lab DHCP).

- Behavioral round-trip against a live BIG-IP VE 17.5: identical load-balancing distribution (0.0% delta), persistence, and health behavior on the migrated Alteon.

- 11 constructs traffic-proven individually on a live Alteon (weights, backup groups, cookie and client-IP persistence, XFF insertion, custom monitors, LOGEXP, drain state, PIP).

- Network Segmentation proven at packet level with tcpdump: VIP service, cross-segment redirect to the segment gateway, and segment isolation.

- 37 defects found and fixed during validation, each locked in by a regression test (79 automated tests). The complete ledger is in DELTAS.md in the repository.

# 10. Troubleshooting

| Symptom | Resolution |
| --- | --- |
| npx: command not found | Node.js is not installed or not on PATH - see Section 2. |
| "No F5 config found" on an archive | The archive does not contain config/bigip.conf. For folders with several device configs, the tool lists what it found so you can pick one. |
| A line is rejected when applying on the Alteon | Check log1: platform-capacity limits (for example, more pools than the target supports) and physical-port mappings are flagged there. Validated target versions are Alteon 34.x. |
| Cookie persistence rejected | The target Alteon needs the cookie-persistence license; the generated pbind cookie insert syntax is correct on licensed units. |
| Windows path issues | PowerShell, CMD, and Git Bash all work; quote paths containing spaces. |

# 11. Resources

| Resource | Location |
| --- | --- |
| npm package | npmjs.com/package/@radware/f5-to-alteon |
| Source and issues | github.com/Radware/F5-to-Alteon-Migrator |
| Delta ledger vs legacy tool | DELTAS.md in the repository |
| Validation reports | validation/VALIDATION_STATUS.md in the repository |

RADWARE  |  PROFESSIONAL SERVICES